Lab Practical #02:

Study of different types of network cables & connectors and practically implement the cross-wired cable and straight through cable using crimping tool.

Practical Assignment #02:

- List various networks cable and connectors. Also, write short description.

- Give cross-wired cable and straight through cable diagram (Color Code wise).

## List various networks cable and connectors. Also, write short description.

- Twisted Pair Cable:

  - Network Cable Type: Guided

  - Description:

  - separately insulated.

  - Twisted together.

  - It is widely used in different kind of data and voice infrastructure.

  - The use of two wire twisted together helps to reduce crosstalk and electromagnetic induction

  - Two types of twisted pair cable:

- UTP (Unshielded twisted pair)

- Ordinary telephone wires

- Less expensive

- We community against noise interferences

- Most use in two categories: Cat-3 & Cat-5

- Used in laboratory

- STP (shielded twisted pair)

- An extra metallic shield on each pair

- Relatively more expensive

- Better performance then UTP

- Used in exterior network(outside of building)

- Coaxial Cable:

  - Network Cable Type: Guided

  - Description:

  - Open conductor is braided shielded.

  - Inner conductor is solid metal.

  - Separated by insulating material, and whole covered by plastic cover.

  - Used in television, long distance telephone transmission.

  - Hi bandwidth an excellent voice immunity.

- Fiber Optic Cable:

  - Network Cable Type: Guided

  - Description:

  - A Fiber-Optic cable is made of glass or plastic and transmit signals in form of lights.

  - Light travel in straight line as long as it is moving through a single uniform substance.

  - It is surrounded by cladding of less dense glass or plastic so, difference in density of two materials must be such that a beam of light moving through the core is reflected off the cladding instead of being reflected into it.

  - It is you reflection to guide light through a channel.

  - Small size and weight

  - Used in high bandwidth network

  - I data rate and lower attenuation.

- RJ45 Connector

- Description:

  - It is one of the most used networking connector in network world.

  - Made by plastic.

  - Larger than a classical telephone connector RJ11.

  - RJ45 is the connector of UTP and STP type cables.

  - With this connectors we can connect the cable between any two network devices, to routers, to switches, firewalls.

- Coaxial Cable Connectors

- Description:

  - Coaxial cable connectors are used to connect cables to other devices and maintain the cable’s shielding.

  - There are two distinct connector styles – known as male and female. Male connectors have metals pin which protrude from the centre and female connectors have a recessed hole to receive the pin.

  - Here are some of the most common coaxial connector types and their applications:

  - BNC Connector:

        - Bayonet Neil-Concelman (BNC) coaxial connectors are used for quick connection or disconnection in RF equipment, test instruments, radio, television, and video signal.

  - TNC Connector:

        - The Threaded Neill-Concelman (TNC) connectors are small, weatherproof products which operate up to 12GHz. They’re often used in mobile phone and RF/antenna connections.

  - SMB Connector:

        - Subminiature version B (SMB) connectors are one of the most popular RF/microwave connectors for industrial and telecommunications equipment – offering a simple snap-on coupling design for cables with uncommon connections.

  - 7/6 DIN Connector:

        - The 7/16 DIN (Deutsches Institut für Normung) connector is a threaded RF connector used to connect coaxial cables.

        - These connectors are mainly used for applications which require water resistance, as they’re robust and durable. They’re particularly useful for base stations and broadcast communication systems.

  - QMA Connector:

        - QMA connectors are coaxial radio frequency connectors. They maintain the shielding barrier in electrical applications and feature snap-on mechanisms. They’re ideal for industrial and communications settings.

  - MCX Connector:

        - Micro coaxial (MCX) connectors are ideal for environments restricted in size or space.

        - MCX connectors operate between DC and 6GHz in wireless, GPS, TV tuner cards, RF hardware, and digital cellular applications. These connectors also have a snap-on coupling design for easy installation.

- Fiber Optic Cable Connectors

- There are four common elements of a fiber optic connector. These elements are:

  - Ferrule

  - Body

  - Coupling

  - Core

- The Ferrule is the structure that covers the fiber.

- The Body is the cover that protects the fiber.

- The Coupling is the cover that prevents signals in the core from escaping.

- The Core is the exact conduit part of the fiber that light passes through.

- Types of Connectors:

  - SC Connector:

- SC (Subscriber Connector) is used  on MMF or SMF. It is developed by Japon NTT . The mechanism used with SC Connector is push on/off mechanism.  It is rerely used in todays network World. Instead of SC Connector, LC and MTP Multi Fiber networking connectors are used.

  - ST Connector:

- ST (Straight Tip Connector) is the first connector type used for fiber optic termination at the end of multi mode cables. It is developed by AT&T. ST Connector is used with twist on/off mechanism.  It is rerely used in todays network World. Instead of ST Connector, LC and MTP Multi Fiber Connectors are used.

  - LC Connector:

- LC (Lucent Connector) is a widely used SFF (Small Form Factor) networking connector used for high-density connections. LC Connector is consist of two fibers connected together at the same time and it is always duplex. It is developed by Lucent Technologies.

## Give cross-wired cable and straight through cable diagram (Color Code wise).

- Cross-wired Cable Diagram (Color Code)

- Straight Through Cable Diagram (Color Code)